Protocol for GPS Watch

V1.0

Communication protocol

Copyright statement

all rights reserved.

Unauthorized copy or distribution of this document, in whole or in part will bear all legal responsibilities.

Revised record:

| 2016/8/8 | Engineer kaka | original version |
| --- | --- | --- |
| 2016/9/8 | Engineer kaka | add multiple bases AP02 and |
| 2016/10/12 | Engineer kaka | add wifi data |
| 2016/12/12 | Engineer Jean | Add voice message |
| 2017-3-22 | Engineer Jean | Add reminder |
| 2017-6-18 | Engineer Jean | Add heartrate Add Blood Pressure |
| 2018-6-29 | Engineer kaka | Add fall down alert Add |
| 2019-5-8 | Engineer kaka | Add new sms command |
| 2021-03-21 | Engineer kaka | Add ECG command |

Content

1. Communication agreement	3

二：Commands sent from end user (device server)	6

1.	AP00-Login package(responds:BP00)	6

2.	AP01 Locating package, GPS+LBS+Status+Base +WIFI combining package (responds:BP01)	6

3.	AP02 Multiple bases locating package (responds:BP02)	7

4.	AP03 Heartbeat package(responds:BP03)	8

5.	AP07 Upload audio message（responds:BP07）	9

6.	AP10 Alarm and Return address Package(responds:BP10)	10

7  AP49 Upload heart rate（responds：BP49）	11

8	APHT  Upload heart rate and BP（responds：BPHT）	12

9	APHP Upload heart rate、BP、SPO2、blood sugar（responds：BPHP）	12

10	AP50 Upload body temperature （responds：BP50）	13

11	ECG upload（upload：APHD，reply：BPHD）	13

三：Commands sent from server（server device）	14

1.	BP12 Set sos numbers(three)（respond：AP12）	14

2.	BP14 Set white list(Phone book) numbers(ten)（respond：AP14）	15

3.	BP16 Real-time locating command（respond：AP16）	16

4.	BP17 Factory reset（respond：AP17）	16

5.	BP20 set timezone（respond：AP20）	17

6.	BP28 Send audio message to device（responds:AP28）	17

1.	BP31 POWER OFF（responds：AP31）	18

7.	BP33 Working Mode（respond：AP33）	19

8.	BP34 Working Mode for free setting（respond：AP34）	19

9.	BP76 fall down switch（responds：AP76）	20

10.	BP77 fall down sensitivity（responds：AP77）	20

11.	BP84 Switch for White list（respond：AP84）	21

12.	BP85 Set an alarm/reminder（respond：AP85）	21

13.	BP86 Set the interval of heart rate auto testing（respond：AP86）	22

14.	BPXL Test heart rate（respond：APXL）	23

15.	BPXY  Test blood pressure（respond：APXY）	23

16.	BPXT  Test temperature（respond：APXT）	24

17.	BP87  set the interval of auto Test temperature（respond：AP87）	24

18.	BPXZ Test blood oxygen（respond：APXZ）	25

1. Communication agreement

1.1 Introduction

This  document  defines  the  GPS  locator  positioning  service  platform  for  the  application  layer interface  protocols.The  relevant  interface  protocols Only  apply  to  the  interaction  between  the  platform and positioning terminal.

1.2 Network connection way

Long connection is through 2G GSM/4G LTE using TCP connection. The relationship between watch and server and app is shown in the figure below. Watch send data(login data and location data and health data) to server by mobile network; Server can reply watch and send command to watch. App and web can get data from server and show to user.

1.3 Terms and definitions

| Terms abbreviations | Meaning |
| --- | --- |
| CMPP | China Mobile Peer to Peer |
| GPS | Global Positioning System |
| GSM | Global System for Mobile Communication |
| GPRS | General Packet Radio Service |
| TCP | Transport Control Protocol |
| LBS | Location Based Services |
| IMEI | International Mobile Equipment Identity |
| MCC | Mobile Country Code |
| MNC | Mobile Network Code |
| LAC | Location Area Code |
| CI | Cell ID |
| RSSI | Received Signal Strength Indicator |
| UDP | User Datagram Protocol |
| SOS | Save Our Ship/Save Our Souls |
| CRC | Cyclic Redundancy Check |
| NITZ | Network Identity and Time Zone, |
| GIS | Geographic Information System |

二：Commands sent from end user (device server)

- AP00-Login package(responds:BP00)

| Example: |  |
| --- | --- |
| IWAP00353456789012345# |  |
| Notice: |  |
| IW: Identifier AP00:Command word 353456789012345: IMEI number of devices, default as 15 digits #:Terminator |  |
| platform responds: | platform replies: IWBP00,20150101125223,8# 20150101125223 is server time service, the format is year month date hour minute second, it’s UTC0 time. 8 is the present time zone of server |
| Notice: | New login package will be sent every time the device connects with server. |

- AP01 Locating package, GPS+LBS+Status+Base +WIFI combining package (responds:BP01)

| Example: |  |
| --- | --- |
| IWAP01080524A2232.9806N11404.9355E000.1061830323.8706000908000102,460,0,9520,3671,Home¦74-DE-2B-44-88-8C¦97& Home1¦74-DE-2B-44-88-8C¦97&Home2¦74-DE-2B-44-88-8C¦97& Home3¦74-DE-2B-44-88-8C¦97# |  |
| Notice: |  |
| IW Identifier AP01: Command word 080524: 24th May 2008 A:“A”shows valid data,”V”shows invalid data and will get LBS data 2232.9806N11404.9355E000.1: 22 degrees north latitude 32.9806 points，114 degrees east longitude 04.9355 points，speed is 000.1 km/h，it’s default as 0 if the latitude and longitude are invalid, such as 0000.0000N00000.0000E 061830: GMT 06:18:30 323.87:direction angle is 323.87° 06000908000102: 060 is GSM signal,009 is the number of satellites,080 is battery level, 0 is remaining space,01 is fortification state,02 is working mode, (it shows none setting if fortification and working mode are 00) 460,0,9520,3671 : LBS is base data MCC is country code,460 means China,0:MNC,0 is moving,9520:LAC,decimal,3671,CID, decimal If state in GPS package is V or latitude and longitude are “ 0000.0000N00000.0000E”,it will get LBS data. Home¦74-DE-2B-44-88-8C¦97 : one set of WIFI information，Home is SSID， 74-DE-2B-44-88-8C is MAC address，97 is signal strength，variables are separated by“¦”,wifi information can be multiple sets and are separated by “&”. |  |
| platform responds: | platform replies: IWBP01# |
| Notice: |  |

- AP02 Multiple bases locating package (responds:BP02)

| Example： |  |
| --- | --- |
| IWAP02,zh_cn,0,7,460,0,9520¦3671¦13,9520¦3672¦12,9520¦3673¦11,9520¦3674¦10,9520¦3675¦9,9520¦3676¦8,9520¦3677¦7,4,1¦D8-24-BD-79-FA-1F¦59&2¦3C-46-D8-6D-CE-01¦81&3¦0C-4C-39-1A-7C-65¦69&4¦70-A8-E3-5D-D7-C0¦65# |  |
| Notice: |  |
| IW Identifier AP02: Command word zh_cn: language notice 0:reply flag，not use. 7: 7 sets of bases 460: MCC is country code 0: MNC operator code 9520¦3671¦10: LAC¦CID¦dbm shows a set of base information and the numbers should correspond to the number of bases. 10：dbm is signal strength, multiple bases signal strength is suggested to be arranged from high to low then downloaded, signal strength are using absolute values: 150 – abs(dbm) 4: sets of wifi 1¦D8-24-BD-79-FA-1F¦59: ssid¦mac address¦ signal strength，signal strength are using absolute values:150 – abs(signal strength) |  |
| platform responds: | platform replies: IWBP02# |
| Notice: |  |

- AP03 Heartbeat package(responds:BP03)

| Example： |  |
| --- | --- |
| IWAP03,06000908000102,5555,30# |  |
| Notice: |  |
| IW: Identifier AP03: Command word Heartbeat message , Device can use this message to keep long connection with server 06000908000102: 060 is GSM signal , not use 009 is the number of satellites 080 is battery level 0 is remaining space 01 is fortification state, Here shows by two hexadecimal,00 if no value, the length should be two digits. There’re 8 digits in total and each digit is defined as below (unlisted one is unused states,1 shows effective and 0 shows invalid) Bit0 fortification state<the device is invalid> Bit1 :1 shows the night-light open,0 shows night-light off 02 is working mode, (it shows none setting if fortification and working mode are 00) 5555: counting steps. Recount from 0 after reaching 99999 steps (9999 is for an example, here is 2 bytes int and overflow to zero) 30: Rolls frequency |  |
| platform responds: | platform replies: IWBP03# |
| Notice: |  |

- AP07 Upload audio message（responds:BP07）

| Example： |  |
| --- | --- |
| IWAP07,20140818064408,6,1,1024,XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX# |  |
| Notice: |  |
| IW：Identifier AP07 : Command word 20140818064408 :yyyyMMddHHmmss ，device record time 6：the total number of audio data packet 1 ：sequence of the audio data packet 1024 :the length of audio data XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX：audio data as bytes, each packet can not beyond 1024 byte，the last audio data packet maybe less than 1024 byte, if less than 1024 byte then send length as actual size The rule for audio upload： 1: if device not received the responds from server, it will repeat upload the same audio data packet 2: if the server responds that received succesfully, device will upload the next audio data packet if current sequence not reach the total packet number 3: if the server responds that received failed, it will repeat upload the same audio data packet |  |
| platform responds: | IWBP07,20140818064408,6,1,1# 20140818064408：device record time 6：the total number of audio data packet 1: current sequence of the audio data packet the sever received 1：the server responds that received succesfully，0:the server responds that received failed |
| Notice: | Now only support amr audio between device and server |

- AP10 Alarm and Return address Package(responds:BP10)

| Example： |  |
| --- | --- |
| IWAP10080524A2232.9806N11404.9355E000.1061830323.8706000908000502,460,0,9520,3671,00,zh-cn,00,HOME¦74-DE-2B-44-88-8C¦97&HOME1¦74-DE-2B-44-88-8C¦97&HOME2¦74-DE-2B-44-88-8C¦97&HOME3¦74-DE-2B-44-88-8C¦97# |  |
| Notice: |  |
| IW : Identifier AP10: Command word 080524: 24th May 2008 A: “A”shows valid data,”V”shows invalid data and will get LBS data 2232.9806N11404.9355E000.1: 22 degrees north latitude 32.9806 points，114 degrees east longitude 04.9355 points，speed is 000.1 km/h，it’s default as 0 if the latitude and longitude are invalid, such as 0000.0000N00000.0000E 061830: GMT 06:18:30 323.87: direction angle is 323.87° 06000908000102: 060 is GSM signal,009 is the number of satellites,080 is battery level, 0 is remaining space,01 is fortification state,02 is working mode, (it shows none setting if fortification and working mode are 00) 460,0,9520,3671 : LBS is base data MCC is country code,460 means China,0:MNC,0 is moving,9520:LAC,decimal,3671,CID, decimal 00 is alarm state,00 is no alarm (01：SOS,03: not wear, 05/06：fall down alarm) zh-cn: device language 00:The first one 0:whether the reply address is needed,0:no reply,1 reply. The second 0:whether mobile hyperlink is contained in address information,0 not contained,1 contained Home¦74-DE-2B-44-88-8C¦97 : one set of WIFI information，Home is SSID， 74-DE-2B-44-88-8C is MAC address，97 is signal strength，variables are separated by“¦”,wifi information can be multiple sets and are separated by “&”. |  |
| platform responds: | platform responds BP10,if return address is not needed, there should be space in return package. IWBP106df157335e0253575c71533a53576d7759279053003100300037003953f7002000200068007400740070003a002f002f007700770077002e006700700073002e0063006f006d002f006d00610070002e0061007300700078003f006c00610074003d00320033002e0031003200330026006c006e0067003d003100310033002e003100320033# UNICODE encoded reply message from server which contains address, above example shows: 深圳市南山区南海大道1079号http://www.gps.com/map.aspx?lat=23.123&lng=113.123 The language and content automatically judge according to langiage inAP10 package, whether to reply hyperlink according to states of AP10. |
| Notice: |  |

7  AP49 Upload heart rate（responds：BP49）

| Example： |  |
| --- | --- |
| IWAP49,68# |  |
| Notice: |  |
| IW：Identifier AP49 : Command word 68 : Heart rate |  |
| platform responds: | IWBP49# |
| Notice: |  |

- APHT  Upload heart rate and BP（responds：BPHT）

| Example： |  |
| --- | --- |
| IWAPHT,60,130,85# |  |
| Notice: |  |
| IW：Identifier APHT : Command word 60 : Heart rate 130: systolic pressure 85 : diastolic pressure |  |
| platform responds: | IWBPHT# |
| Notice: |  |

- APHP Upload heart rate、BP、SPO2、blood sugar（responds：BPHP）

| Example： |  |
| --- | --- |
| IWAPHP,60,130,85,95,90,,,,,,,,# |  |
| Notice: |  |
| IW：Identifier APHT : Command word 60 : heart rate 130:blood pressure[SBP] 85 : blood pressure[DBP] 95：SPO2 90：blood sugar |  |
| platform responds:： | IWBPHP# |
| Notice： | Each value is not necessary.if there is no value, the packet leave it blank |

- AP50 Upload body temperature （responds：BP50）

| Example： |  |
| --- | --- |
| IWAP50,36.7,90# |  |
| Notice: |  |
| IW：Identifier AP50 : Command word 36.7 : body temperature 90：device battery level |  |
| platform responds:： | IWBP50# |
| Notice： | Each value is not necessary.if there is no value, the packet leave it blank |

- ECG upload（upload：APHD，reply：BPHD）

| Example： |  |
| --- | --- |
| IWAPHD,20200707111800,6,1,1.3,20.2,1,1000,1024,XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX# |  |
| Notice: |  |
| IW：Identifier APHD: Command word 20200707111800: date and time; 2020Y 7m 7d 11h8m00s 6: 6 data packets 1：now it is the first data packet 1.3: gain 20.2：0 point voltage 1：Lead number 1000： sampling rate 1024：Data length，max is 1024 XXXXXXXXXXXXXX: Ecg data |  |
| Server reply： | IWBPHD,0, 98, 59, 156, 296, 369,7aa660275fc35f8b4e0d9f50¦623f98a4 # 0：AI analysis OK 1：AI analysis NG 98: average heart rater 59: Qrs 156: PR 296: Q-t 369: QTc 7aa660275fc35f8b4e0d9f50¦623f98a4：atrial fibrillation |
| Notice: |  |

三：Commands sent from server（server device）

- BP12 Set sos numbers(three)（respond：AP12）

| Example： |  |
| --- | --- |
| IWBP12,353456789012345,080835,135XXXXXXXX,135XXXXXXXX,135XXXXXXXX# |  |
| Notice: |  |
| IW：Identifier BP12 : Command word 353456789012345:IMEI number and the unique ID of the device 080835 : journal no. 135XXXXXXXX,135XXXXXXXX,135XXXXXXXX : SOS numbers,max set 3 SOS, all the phone no. should be set one by one , If one of the SOS not be set, the corresponding position leave space. |  |
| device responds： | IWAP12,080835,135XXXXXXXX,135XXXXXXXX,135XXXXXXXX# 080835: The device responds with response journal no. |
| Notice: |  |

- BP14 Set white list(Phone book) numbers(ten)（respond：AP14）

| Example： |  |
| --- | --- |
| IWBP14,353456789012345,080835,D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx# |  |
| Notice: |  |
| IW：Identifier BP14 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. D3590D54¦135xxxxxxxxxx : A set of contacts and use ¦ to separate name and phone number, names encode UNICODE and different sets are separated by a comma, If a phone number is not set, the corresponding position leave space. |  |
| device responds： | IWAP14,080835, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx# 080835: The device responds with response journal no. |
| Notice: |  |

- BP16 Real-time locating command（respond：AP16）

| Example： |  |
| --- | --- |
| IWBP16,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP16 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAP16,080835# 080835: journal no. device will Async send AP01 location data after responds. |
| Notice: |  |

- BP17 Factory reset（respond：AP17）

| Example： |  |
| --- | --- |
| IWBP17,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP17 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAP17,080835# 080835: journal no. |
| Notice: |  |

- BP20 set timezone（respond：AP20）

| Example： |  |
| --- | --- |
| IWBP20,353456789012345,080835,0,8# |  |
| Notice: |  |
| IW：Identifier BP17 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 0: language ,not use 8: timezone, eastern time zone great than zero , west time zone great than zero ,for example -8, now only supports integer time zones |  |
| device responds： | IWAP17,080835# 080835: journal no. |
| Notice: |  |

- BP28 Send audio message to device（responds:AP28）

| Example： |  |
| --- | --- |
| IWBP28, D3590D54,XXXX,6,1,1024,XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX# |  |
| Notice: |  |
| IW：Identifier BP28 : Command word D3590D54: UNICODE，the sender name XXXX: additional data 6：the total number of audio data packet 1 ：sequence of audio data packet 1024 :the length of data packet XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX：audio data, each audio can not beyond 1024 byte，the last audio data packet maybe less than 1024 byte, if less than 1024 byte then send length as actual size The rule for audio upload： 1: if server don’t received the responds from device, it will repeat send the same audio data packet 2: if the device responds that received succesfully, it will send the next audio data packet 3: if the device responds that received failed, it will repeat send the same audio data packet |  |
| device responds： | IWAP28,D3590D54,XXXX,6,1,1# D3590D54:UNICOD，sender name XXXX: additional data,return the same content received from the server 6：the total number of audio data packet 1：sequence of the audio data packet the device received 1：device have received the audio data，0:device don’t have received the audio data |
| Notice: | Now only support amr audio between device and server |

- BP31 POWER OFF（responds：AP31）

| Example： |  |
| --- | --- |
| IWBP31,353456789012345,080835# |  |
| Notice： |  |
| IW：Identifier BP31 : Command word 353456789012345: IMEI number and the unique ID of the device 080835: journal no. |  |
| device responds： | IWAP31,080835# 080835: journal no. |
| Notice： |  |

- BP33 Working Mode（respond：AP33）

| Example： |  |
| --- | --- |
| IWBP33,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP33 : Command word 353456789012345: IMEI number and the unique ID of the device 080835: journal no. 1：working mode of the device， 1：normal mode，every 15minute a position report with WIFI and lbs 2：Power-saving mode，every 60 minute a position report with WIFI and lbs 3：emergency mode 1 minute a position report with GPS and WIFI and lbs |  |
| device responds： | IWAP33,080835,1# 080835: journal no. |
| Notice: |  |

- BP34 Working Mode for free setting（respond：AP34）

| Example： |  |
| --- | --- |
| IWBP34,353456789012345,080835,8,30,1# |  |
| Notice: |  |
| IW：Identifier BP34 : Command word 353456789012345: IMEI number and the unique ID of the device 080835: journal no. 8: mode 8 30 seconds is the interval of the location report 1: is mean GPS is ON and if it is 0 mean GPS off |  |
| device responds： | IWAP34,080835,1# 080835: journal no. |
| Notice: |  |

- BP76 fall down switch（responds：AP76）

| Example： |  |
| --- | --- |
| IWBP76,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP76 : Command word 353456789012345: imei 080835: journal no. 1/0: enable/disable |  |
| platform responds: | IWAP76,080835# |
| Notice: |  |

- BP77 fall down sensitivity（responds：AP77）

| Example： |  |
| --- | --- |
| IWBP77,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP77 : Command word 353456789012345: imei 080835: journal no. 1/2/3: level 3 is the most sensitive |  |
| platform responds: | IWAP77,080835# |
| Notice: |  |

- BP84 Switch for White list（respond：AP84）

| Example： |  |
| --- | --- |
| IWBP84,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP84 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 1: white list on 0: white list off |  |
| device responds： | IWAP84,080835,1# 080835: journal no. 1：device have run the command , 0：device don’t have run the command |
| Notice: |  |

- BP85 Set an alarm/reminder（respond：AP85）

| Example： |  |
| --- | --- |
| IWBP85,353456789012345,080835,1,3, 0900,135,1,1@0900 ,135,1,2@0900 ,135,1,3# |  |
| Notice: |  |
| IW：Identifier BP85 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 1: master switch for all alarm，1 is on，0 is off 3: total number of all alarm 0900 ,135,1,1 : a reminder，0900 mean reminder time 09:00, 135 mean Monday,Wednesday,Friday ，24 hour system 1 switch for this alarm 1 is on 0 is off 1 Take the medicine reminder 2 Drink water reminder 3 A sedentary reminder Each alarm Separated by ”@” |  |
| device responds： | IWAP85, 080835,1,3, 0900,135,1,1@0900 ,135,1,2@0900 ,135,1,3# 080835:The device responds with response journal no. |
| Notice: |  |

- BP86 Set the interval of heart rate auto testing（respond：AP86）

| Example： |  |
| --- | --- |
| IWBP86,353456789012345,080835,1,720# |  |
| Notice: |  |
| IW：Identifier BP86 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 0: close measurement; 1: turn on auto measurement; 720 is 720minutes |  |
| device responds： | IWAP86,080835# 080835:The device responds with response journal no. |
| Notice: |  |

- BPXL Test heart rate（respond：APXL）

| Example： |  |
| --- | --- |
| IWBPXL,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXL : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXL,080835# 080835:The device responds with response journal no. |
| Notice: |  |

- BPXY  Test blood pressure（respond：APXY）

| Example： |  |
| --- | --- |
| IWBPXY,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXY : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXY,080835# 080835:The device responds with response journal no. |
| Notice: |  |

- BPXT  Test temperature（respond：APXT）

| Example： |  |
| --- | --- |
| IWBPXT,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXT : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXT,080835# 080835:The device responds with response journal no. |
| Notice: | This command used to test once temperature |

- BP87  set the interval of auto Test temperature（respond：AP87）

| Example： |  |
| --- | --- |
| IWBP87, 353456789012345, 080835,1,720# |  |
| Notice: |  |
| IW：Identifier BP87 : Command word 353456789012345: IMEI number and the unique ID of the device 1 mean on the auto test of temperature; 0 means off the auto test of temperature 720 means the interval is 720 minutes 080835 : journal no. |  |
| device responds： | IWAPXT,080835# 080835:The device responds with response journal no. |
| Notice: | This command used to test once temperature |

- BPXZ Test blood oxygen（respond：APXZ）

| 示例： |  |
| --- | --- |
| IWBPXZ,353456789012345,080835# |  |
| 说明： |  |
| IW：Identifier BPXZ : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| 设备响应： | IWAPXZ,080835# 080835: The device responds with response journal no. |
| 说明： |  |

三：Some questions and answers

SMS command：

| Set server’s ip and port |  |
| --- | --- |
| Format | #host#=Domain name,port# or #IP#=ip address,port# |
| Example | #host#=api.gswatch.com,5088# or #ip#=198.11.183.28,5088# |
| Reply sms | Host set ok or IP set ok |
| Add a new apn |  |
| Format | #apn#=mcc,mnc,APNname,accname,usrname,pwd,auth_type# |
| Example | #apn#=460,00,cmnet,cmnet,,,PAP# |
| Reply sms | Apn set ok |
| Check imei |  |
| Format | #status# |
| Example | #status# |
| Reply sms | APN:cmnet;SOS NUM:,,;DOMAIN:l05.personalalarmwatch.com,8090;IP IN USE:118.178.155.223,8090;IMEI:865513041152601;BATTERY:025%;FALL DOWN:1;SW version:L05_W002_V1.75_EVAN_7068_7981_20211015 10-16-2021 12:01:07 |
| Set time zone |  |
| Format | #timezone |
| Example | #timezone:-8.00 |
| Reply sms | timezone: -8.00 ok |
| Set time for watch: if you want set watch time by sms, you should off getting time from server first |  |
| Format | #timedate: |
| Example | #timedate:2019.06.18,19:24 |
| Reply sms | #timedate:2019.06.18,19:24 ok |